TROLLEY WHEEL LOAD DATA

Hoist 1

1	General

Hoist load (SWL) 	5 000  kg

Hoisting height  	6,0 m [max 6,00 m]

Hoist type  	SH 4012-20 4/1 L2

Hoist group  	M5

Trolley traversing duty group	M5

Hoisting speed  	5/0,83 m/min 	2-speed

Trolley traversing speed	20/5 m/min	2-speed

Dynamic coefficient for live load	1,19

Dynamic coefficient for dead load	1,10

Sideways pull factor	1,05

Weight of hoist and trolley	330 kg

2	Wheel load drawing

3	Wheel load data

Distance between trolley wheels (L15)	570 mm

Distance between trolley wheels (L30)	570 mm

Distance between trolley wheels (L59)	 mm

Girder flange width (B__)	260 mm

Trolley wheel contact point from flange edge (B1_)	18 mm

Static Wheel load (load+trolley /trolley only) (NR1)	1 498 kg	/ 99 kg

Static Wheel load (load+trolley /trolley only) (NR2)	1 649 kg	/ 66 kg

Static Wheel load (load+trolley /trolley only) (NR3)	1 498 kg	/ 99 kg

Static Wheel load (load+trolley /trolley only) (NR4)	1 649 kg	/ 66 kg

Dynamic Wheel load (load and trolley wgt.) (NR1)	17,2 kN

Dynamic Wheel load (load and trolley wgt.) (NR2)	19,4 kN

Dynamic Wheel load (load and trolley wgt.) (NR3)	17,6 kN

Dynamic Wheel load (load and trolley wgt.) (NR4)	19,4 kN