MOTOR DATA

1	General

Crane type	SHS5t x 4,5m Hol:6m

Span	4,500 m

Crane load (SWL)	5 000 kg

Crane voltage	380 V

2	Hoisting motor

SH 4012-20 4/1 L2

Hoist 1

Motor code 	H42

Motor type 	1 x H42

Synchronized speed 	2 750 / 415 r/min

Duty group 	M5

Max. nominal power 	0,7/4,5 kW

Rated power	0,7/4,5 kW

Duty factor 	20/40 % ED

Starting torque (High Speed) 	44,0 Nm

Starting / Nominal current (High Speed) 	46,3 / 10,4 A

Power factor at starting/nominal current 	0,84 / 0,92

3	Trolley traversing motor

Hoist 1

Motor type	1 x 8/2F12

Motor nominal/supply voltage	 /  V

Synchronized speed	 /  r/min

Nominal speed	2 420 / 590 r/min

Duty group	M5

Nominal power	0,37/0,1 kW

Duty factor	40 / 20 % ED

Starting torque (High Speed)	3,6 Nm

Starting / Nominal current (High Speed)	3,0 / 1,2 A

4	Bridge traveling motor

Motor type	8/2F12

Motor voltage	380 V

Number of motors	2 ps

Synchronized speed	3 000 / 750 r/min

Nominal speed	2 420 / 605 r/min

Motor speed with nominal load	2 672 r/min

Duty group	M5

Nominal power	0,37/0,09 kW

Duty factor	40 % ED

Starting torque (High Speed)	3,6 Nm

Starting current / Nominal current(High Speed)	3,4 / 1,4 A